Mine Tailings Cover

Percolation Estimator — Quickstart Guide

CIVE 438 · Fundamentals of Environmental Engineering · Spring 2026

# Overview

This tool estimates the annual percolation rate through a single-layer mine tailings cover using a simplified lumped water balance. It combines the SCS Curve Number method with Darcy's Law to determine how much precipitation ultimately passes through the cover and reaches the tailings below. The result is compared to a user-selected regulatory threshold and flagged as Pass, Marginal, or Fail.

# Inputs

Open the HTML file in any web browser. At first, the page will only have inputs. Enter values appropriate for your site before clicking “run water balance model.”

| Input Parameter | Units | Default / Range | Description |
| --- | --- | --- | --- |
| Annual Precipitation (P) | mm/yr | 450 (200–5000) | Total annual rainfall + snowmelt at the site. Use long-term average from NOAA or local gauge. |
| SCS Curve Number (CN) | dimensionless | 70 (1–100) | Runoff potential of the cover surface. Bare/compacted soil ≈ 80; well-vegetated cover ≈ 55–70. |
| Annual ET | mm/yr | 320 (0–3000) | Evapotranspiration lost from the cover. Arid sites: 200–500 mm/yr; humid: 500–900 mm/yr. |
| Hydraulic Conductivity (K) | mm/yr | 3.15 (0.001–31536000) | Saturated K of the cover soil. Compacted clay ≈ 3 mm/yr; sandy loam ≈ 31,536 mm/yr. |
| Cover Thickness | m | 1.0 (0.1–10) | Physical depth of the cover layer. Typical mine covers: 0.5–2.0 m. |
| Hydraulic Gradient (i) | m/m | 1.0 (0.01–10) | Driving head across the cover. Use 1.0 (unit gradient) as a conservative default for a saturated cover. |
| Percolation Threshold | mm/yr | 60 (preset options) | Regulatory or design target. Options: 30 (stringent), 60 (moderate), 150 (relaxed), or custom. |

# How the Tool Works

The tool executes a three-step water balance, then compares the result to the cover's hydraulic capacity:

- Runoff via SCS-CN Method

S=25,400CN-254#1

Where S is soil retention in mm.

Q=P-0.2∙S2P+0.8∙S#2

Annual precipitation (P) is split into surface runoff (Q, mmyr) and net infiltration (P − Q) using the curve number. A higher CN produces more runoff.

- Evapotranspiration Removal

∆W=max0, Infiltration-ET#3

The tool subtracts annual ET from net infiltration. If ET exceeds infiltration, no water is available for percolation (ΔW = 0). This is the supply term.

- Darcy's Law — Cover Hydraulic Capacity

q=K∙i#4

Where q is specific discharge (mmyr) and represents the maximum rate the cover can transmit water downward under the given hydraulic gradient. This is the capacity term.

- Percolation — the Limiting Term

Percolation = min(ΔW, qₛ)

Percolation=min∆W, q#5

Actual percolation is whichever is smaller: the water available after ET (supply-limited) or the specific discharge of the cover. If the cover K is very low, the cover limits flow even when water is available. If K is high, water supply controls.

# Outputs

| Output | Units | What It Means |
| --- | --- | --- |
| Runoff (Q) | mm/yr | Water that runs off the surface without infiltrating, calculated via the SCS-CN method. |
| Net Infiltration | mm/yr | Precipitation minus runoff. This is the water that enters the cover. |
| Water Available for Percolation | mm/yr | Infiltration minus ET. Water remaining after evapotranspiration is removed. If negative, ET exceeds infiltration and percolation = 0. |
| Darcy Flux (q = K·i) | mm/yr | The maximum rate the cover soil can transmit water downward. Set by soil hydraulic properties. |
| Estimated Percolation | mm/yr | The lesser of water available and Darcy flux — whichever limits flow controls the result. |
| Verdict (Pass / Warn / Fail) | — | Compares estimated percolation to the selected threshold. Includes a plain-language explanation of the controlling factor. |
| Water Balance Bar Chart | % of P | Visual breakdown showing how precipitation is partitioned into runoff, ET, percolation, and soil storage. |

# Interpreting Results

## Verdict Flags

- Pass: Percolation is ≤75% of the threshold. The cover design is meeting the target with margin.

- Marginal / Warn: Percolation is 75–100% of the threshold. The design is borderline; apply a safety factor or refine inputs.

- Fail: Percolation exceeds the threshold. The cover design requires modification.

## Diagnosing What Controls Percolation

The verdict message tells you which mechanism is controlling:

- Supply-limited: To further reduce percolation, increase vegetative ET or surface CN. Reducing K alone will not help if water supply is already the bottleneck.

- Capacity-limited: To reduce percolation, reduce the hydraulic conductivity of the cover soil, or add a geomembrane layer. The current soil is transmitting water faster than the water balance can remove it through other pathways.

## Threshold Selection Guidance

- 30 mm/yr: Use for acid-generating tailings, wet climates, or sites near sensitive water bodies where leachate is highly harmful.

- 60 mm/yr: General default used in many state and EPA guidance documents for mine cover design.

- 150 mm/yr: Appropriate only for arid sites where low precipitation naturally limits flux, or for low-risk tailings.

# Important Limitations

This tool uses a simplified annual water balance. Results are estimates and should not substitute for a full site study or regulatory-approved modeling. Key simplifications to be aware of:

- Annual averaging: Seasonal variation, storm events, and snowmelt pulses are not captured. A single large storm can drive percolation that the annual model underestimates.

- ET is assumed fully available: In reality, ET is often limited by soil moisture. In dry years, the tool may overestimate ET removal.

- No preferential flow: Macropores, freeze-thaw cracks, and settling-induced cracks can dramatically increase percolation beyond what Darcy's Law predicts.

- Uniform cover assumed: The tool models one homogeneous layer. Multi-layer covers (e.g., drainage blanket + clay barrier) require a more complex analysis.

- Static hydraulic gradient: Real gradient varies with moisture content and saturation state.